| Cloud RAN 16 System Architecture Specification |  |
| --- | --- |
| Architecture Specification |  |

| Author: | Tero Kinnunen | Approver: | xxx |
| --- | --- | --- | --- |
| ___________________________ Copyright © 2014 Nokia. All rights reserved. No part of this publication may be copied, distributed, transmitted, transcribed, stored in a retrieval system, or translated into any human or computer language without the prior written permission of Nokia. The manufacturer has made every effort to ensure that the instructions contained in the documents are adequate and free of errors and omissions. The manufacturer will, if necessary, explain issues, which may not be covered by the documents. The manufacturer's liability for any errors in the documents is limited to the correction of errors and the aforementioned advisory services. The documents have been prepared to be used by professional and properly trained personnel, and the customer assumes full responsibility when using them. The manufacturer welcomes customer comments as part of the process of continual development and improvement of the documentation in the best way possible from the user's viewpoint. Please submit your comments to the nearest Nokia sales representative. Nokia and the Nokia logo are registered trademarks of Nokia. Other company and product names mentioned herein may be trademarks or trade names of their respective owners. |  |  |  |  |

# Revision history

| Version | Date | Modified by | Status | Accepted by | Main changes |
| --- | --- | --- | --- | --- | --- |
| 0.0 |  | Tero Kinnunen | Draft |  |  |
| 0.1 | 30.12.2015 | Tero Kinnunen | First public version |  |  |

IMS Location:

https://sharenet-ims.int.net.nokia.com/livelink/livelink?func=ll&objId=539155485&objAction=browse&viewType=3

Contents

1	Revision history	2

2	Introduction	4

2.1	Scope and purpose	4

3	System Architecture Overview	4

4	CloudBTS in system architecture	5

5	External information model	6

6	External interfaces	7

6.1	LTE telecom interfaces	7

6.2	M-plane interfaces	7

6.3	VNF-RAP interfaces	7

6.4	Cloud management interfaces	8

7	CloudBTS life-cycle management	8

7.1	Onboarding	8

7.2	Deployment	9

7.2.1	VNF deployment	9

7.2.2	CloudBTS creation in NetAct	9

7.2.3	Commissioning and integration of the RAP	10

7.3	Scaling	10

7.4	CloudBTS upgrade/update	11

7.4.1	VNF SW update	11

7.4.2	RAP SW update	11

7.4.3	BTS profiles	11

8	Front-haul transport	12

9	Radio network management functionality	12

10	Terms and Abbreviations	13

11	References	15

# Introduction

## Scope and purpose

This document specifies the Operations and Maintenance related integration of a CloudBTS implemented in CloudBTS16.3 to the radio network and its management system. Potential architecture changes in CloudRAN 17 are excluded. The CloudBTS16.3 is a trial release and split to two separate PT releases, which are both covered in this document.

This architecture document complements the CloudBTS Architecture Specification [CloudBTSArch], which describes the CloudBTS16.3 SW.

This document covers the basic principles for the network management operations for a CloudBTS, external CloudBTS management interfaces, and the principles related to roll-out of a new CloudBTS to the LTE network. In this release WCDMA and GSM solutions are excluded.

Since the CloudBTS is a new product, and upgrade and migration from a bare-metal LTE BTS is not included.

The architecture document does not go to the details of the data center and infrastructure of a radio cloud, although some requirements for the NCIR and CAM can be derived from this specification according to features defined for the release. In this release only the basic NCIR and CAM functionality is expected, and the more advanced features, mainly for operability area, are planned for the following commercial releases.

# System Architecture Overview

Figure – System Architecture Overview

The CloudBTS 16.3 introduces a new architecture where part of the BTS functionality is moved from the physical hardware on the BTS site to cloud in the radio cloud data center. Although the complete end-to-end architecture includes also the neighboring eNBs, LTE core and UE functions, these functions are not impacted and the same external interfaces are used as in a traditional bare-metal LTE BTS deployment.

While this document concentrates on the system level impacts of the new architecture, the detailed functional split between the BTS virtual network function (VNF) and radio access point (RAP) is not covered on details. However, since these two BTS entities are physically distributed, the system level consequences of the front-haul interface are covered here.

The main system level architectural differences compared to bare-metal deployment comes from the radio network data center for the virtual network functions and the Cloud Application Manager (CAM) controlling the life-cycles of the BTS VNFs with the help of services and hardware in the Nokia Cloud Infrastructure for Radio (NCIR).

The same radio network management architecture with NetAct and NE3S interface is used as in the SRAN 16 and SBTS. In trial release only part of the management functions are required.

In this trial release, no interface between the NetAct and CAM is required. As the figure above shows, this means there will also be two separate user interfaces required for managing all aspects of the CloudBTS – NetAct is used for the traditional radio network management while CAM is used for the BTS VNF life-cycle operations.

# CloudBTS in system architecture

In the CloudBTS16.3 trial release the BTS services are split so that part of the functionality is running in the FZM BTS HW, and part in the cloud. The FZM is later in this document referred as Radio Access Point (RAP), and the part running in cloud as CloudBTS Virtual Network Function (VNF).

The CloudBTS VNF is deployed in a data center composed of Nokia IT HW AirFrame servers. Other server types are not supported in the trial release.

The CloudBTS VNF functionality is further split to VNF components (VNFC) running on virtual machines (VM). Later on in this document these components are called as VMs.

Figure – CloudBTS Architecture Overview

Each CloudBTS VM includes a Radio Cloud Platform (RCP) component, which interfaces with the underlying HW and infrastructure. The details of the RCP services used in the CloudBTS are described in the CloudBTS architecture document [CloudBTSArch].

In the trial release the central VMs for the CloudBTS do not scale in the cloud. The only scaling capability of the CloudBTS is in the Baseband (BB) VM, which can be scaled in and out depending on the number of cells in the configuration. The only supported RAP in the trial release is the FZM providing one cell each. Traffic based scaling is not supported.

# External information model

As in the SBTS, also in the CloudBTS the external information model is composed of portion for the common model (SBTS model), and for the RAT specific model (LTE model).

Figure – High level CloudBTS external information model

The common information model is inherited from the SBTS with some changes specific to the cloud deployment. The Radio Access Points (RAP) are modelled as separate object types in the SBTS, and the because of the front-haul for the RAP, also the transport model differs from the model used in the bare metal SBTS.

The LTE model is based on the LTE FLC16A information model used in the LTE FZM (LNBTS/LNCEL)

The external information model will be specified with more details in the CFAM phase.

The relations between the LNCEL and RAP objects in the site configuration is described in the BTS profile. In CloudBTS the BTS profile does not define the HW configuration of the VNF in cloud, but the configurations of the RAPs – e.g. how many and of which type of RAPs the configuration supports, although in the trial only FZM RAP type is supported.

# External interfaces

## LTE telecom interfaces

LTE telecom related external interfaces of the BTS are not changed in the CloudBTS architecture. A connected core network entity or a neighboring eNB does not know if the BTS is deployed as a bare-metal or as a cloud BTS.

## M-plane interfaces

While the CloudBTS O&M is based on the SBTS SiteOAM and NodeOAM architectures, the M-plane interfaces of the CloudBTS are identical to those of the bare-metal SBTS. The functional restrictions in the NE3S interface are described in later chapters of this document.

Although the adaptation to the NE3S functions in NetAct are based on the version of the BTS, which in CloudBTS are is different than that of the bare-metal SBTS, the NE3S manager in NetAct mediates the CloudBTS in the same way as it mediates the management operations of a bare-metal SBTS.

## VNF-RAP interfaces

The VNF-RAP interface includes the REST API communication between the O&M agent in the RAP and the O&M service in the VNF as well as the SysCom communication for the control and user plane.

The REST API is used for setting up and supervising the VNF-RAP communication, while SysCom is used for majority of the rest of the O&M traffic between the VNF and RAP.

In the trial release there is only one O&M agent in the RAP, meaning one single REST API in this interface, but there can be several SysCom communication channels established between the RAP and the VMs of the CloudBTS VNF.

While the communication within this interface is internal to the CloudBTS, the details of this interfaces are out of the scope of the system architecture. However, since the interface is used between two geographically distributed entities, the interface will have some system level visibility.

In the trial release the VNF will be the active entity initially establishing the VNF-RAP interfaces. The VNF uses the VNF-RAP interface for synchronizing the RAP SW version and configuration with the version and configuration of the VNF, and collecting the measurements and fault observations from the RAPs.

## Cloud management interfaces

VeNf–Vnfm

This interface is used for CloudBTS VNF and CAM communication. For this the CAM runs CloudBTS specific scripts in the CloudBTS artefacts, which uses communication plugins, like RestAPI for using the O&M services of the CloudBTS. The used services can either be for SET operations or actions to be executed in the CloudBTS VNF, or GET operations to collect information from the CloudBTS VNF.

The actual communication in this interface is considered as BTS internal messaging without backward compatibility requirements, while the CloudBTS scripts in CAM are CloudBTS version specific.

The operations in this interface can be used for setting up the initial configuration in the VNF deployment. Same mechanisms are typically used also for collecting VNF measurement for scaling decisions, or for initiating graceful shutdown in scale-in or un-deployment, but these are not required in the CloudBTS trial release.

Vi–Vnfm

This interface is used by CAM to initiate VNF operations of the OpenStack in the NCIR. While the CAM is responsible for the cloud management, the actual cloud operations are executed in the NCIR.

Vn–Nf

This is the interface between the infrastructure layer if the cloud and the VNF application. This interface can be used e.g. for reporting errors in the infrastructure which the VNF must take to account in its recovery actions. The Radio Cloud Platform (RCP) hides the actual interface form the CloudBTS application part.

# CloudBTS life-cycle management

The CloudBTS VNF life-cycle operations are done manually using the operations in the CloudBTS application template in CAM. In the CloudBTS 16 release no interactions between the BTS configuration actions in NetAct and life-cycle actions in CAM are required.

## Onboarding

In the onboarding operation the user downloads the CloudBTS SW package from Nokia services, and installs the package to network so that CloudBTS deployment is enabled. In releases beyond the CloudBTS trial the onboarding functionality will be automated, but in the trial release all related steps are manual.

The onboarding includes tasks for installing the CloudBTS images in the CloudBTS SW package to the file service in the NCIR infrastructure of the radio cloud. In addition to this the user installs the CloudBTS artefacts, meaning templates and scripts in the CloudBTS SW package to CAM data storages.

After installing the related files, the user introduces the new CloudBTS SW package with file locations to the CAM, which adds the installed CloudBTS SW package for use. After this the CloudBTS life cycle operations in its application template becomes available in the CAM user interface.

## Deployment

### VNF deployment

The deployment operation is started when the user selects the ‘deploy’ operation in CAM. CAM uses the OpenStack services in NCIR to create the VMs defined in the CloudBTS application template.

In the trial release the CloudBTS VNF deployment includes all VMs required for a fully functional LTE eNB. The deployment includes the VMs for the central functions as well as one baseband VM. The ‘fully functional’ here does not mean the CloudBTS would immediately be ready for LTE user traffic, because this would require also the RAPs in the VNF configuration to be integrated, which naturally will require separate installation and integration steps in addition to the VNF deployment.

In the deployment the user is prompted to insert values for the mandatory parameters for the CloudBTS deployment. Such parameters can be e.g. the host IP address of the CloudBTS VNF used for the external M-plane connection. The configuration in this phase includes only the parameters enabling the VM instantiation and making the VNF visible in NetAct. The final configuration will later on be downloaded from NetAct or with the WebUI of the CloudBTS once the VNF is up and running.

Similarly, an ‘un-deploy’ operation is required in the CloudBTS application template. This operation is used to remove the VNF from the cloud. As in the deployment operation, also here the user needs to perform a separate operation in NetAct to remove the related SBTS element from NetAct data storages.

### CloudBTS creation in NetAct

This step is valid only for the PT2 phase of the CloudBTS16 trial release. In PT1 phase only CloudBTS Web UI is used for the CloudBTS configuration and supervision.

While in the CloudBTS trial release the communication between the NetAct and CAM is not required, creating a new CloudBTS to the network requires a separate user action in NetAct. In this step the user uses the NetAct Monitor user interface to create a new SBTS entity in the NetAct.

The agent information of the CloudBTS in the SBTS creation is identical to that of the bare-metal SBTS. The user inputs the FQDN of the M-plane agent of the SBTS, and adds this FQDN to the NetAct DNS service with the same IP address that is given for the M-plane communication in the VNF deployment.

The order of creation of the SBTS in NetAct, and the deployment of the CloudBTS VNF in CAM is not meaningful, although both steps must be completed before the final radio configuration can be downloaded from NetAct to the CloudBTS.

### Commissioning and integration of the RAP

In CloudBTS16 trial release the auto-connection of the RAPs to the CloudBTS VNF is not supported.

In order to connect a new RAP to the CloudBTS VNF, the RAP needs to be first upgraded with a SW compatible with the CloudBTS SW. New RAP deliveries for the CloudBTS trial will be pre-upgraded in the Nokia delivery chain with a compatible SW, but in case the operator is using FZM devices delivered earlier for the trial, the user must first restart the FZM to factory default mode, and upgrade the SW to CloudBTS compatible version. The CloudBTS compatible RAP SW is delivered to customer within the CloudBTS SW package.

Only RAPs with CloudBTS compatible SW are able to establish the communication channels with the VNF of the CloudBTS.

Once the RAP is installed, the user uses the RAP UI to configure the basic configuration for the RAP, like the host IP address of the RAP. In the trial release the RAP does not have to be made aware of the VNF it needs to connect to, because in trial release the VNF will be the active party for establishing the RAP/VNF communication.

The site configuration (SCF) of the CloudBTS includes a new object class for the RAP configuration. In the trial release the configuration includes e.g. the IP address of the O&M agent of the RAP. The CloudBTS VNF with a RAP configuration attempts to connect to the O&M agent of the RAP, and once successful, the VNF will first check the RAP SW version against its own version, and update the RAP SW is needed – the SW versions needs to match, while there is no backward compatibility requirements for the CloudBTS internal interfaces.

After the communication with RAP is ensured, the CloudBTS VNF will automatically feed the user and control plane communication related configuration and needed cell configuration to the RAP.

The RAP does not have any direct interface with NetAct, but all communication to RAP is controlled in the CloudBTS VNF.

## Scaling

In the CloudBTS16 trial release the SW architecture of the CloudBTS enables only sale in and scale out operations of the baseband (BB) VMs. The scaling operations are initiated as manual user operations in CAM. Measurement based automatic scaling with CAM’s Telco Elasticity Manager (TEM) is not required.

While the initial deployment configuration already includes one BB VM in the setup, the scaling operation is required only when the BB VM is fully allocated with configured cells. When a new cell is configured and the related RAP is connected to the VNF, and there is no room in the already existing BB VMs, the CloudBTS sets an alarm indicating a scaling action is required.

The Scale-out operation itself is manually initiated using the ‘BB scale-out’ operation of the CloudBTS application template in the CAM. CAM uses the NCIR services to instantiate a new BB VM for the VNF. Once the new BB VM is connected to the CloudBTS VNF service, the BTS automatically allocated cells and RAPs without BB allocations to this new BB VM, and clears the related alarm.

In later releases the scale-out actions of the CloudBTS are automated so that a separate user action in CAM is not required, but they are based on the VNF measurements collected in CAM.

Similarly to the scale-out operation, the user may also request scale-in operation for a BB VM. In this operation the user is expected to use the CloudBTS Web UI to gracefully move the cell allocations to another BB VM before initiating the scale-in operation. Once the BB VM is cleared from cell allocations, the user initiates the ‘BB scale-in’ operation in CAM.

In case the BB VM was not cleared form the cell allocations before the scale-in operation, the CloudBTS recovery functions will attempt to re-allocate the cell resources to other available BB VMs.

In later releases the scale-in operations are synchronized with the CloudBTS resource allocations so that no separate user action will be required in the VNF for gracefully shutting down the services using the scaled in VM.

It should be noted that in the trial release the cells are allocated to BB VMs only when the related RAP for the cell is connected and available for the cell service.

## CloudBTS upgrade/update

### VNF SW update

In cloud deployment the SW management of the BTS is not controlled with NetAct SWM applications, but with the CAM. In the SW upgrade the new SW images and artefacts are installed (see onboarding) to CAM and NCIR, and the VMs of the VNF are created newly with these new SW images.

The new SW package of the CloudBTS includes SW for converting the old configuration with the required changes in the new SW. It is assumed that in the trial release these changes will be minimal, and the first real data conversion will be required when the first commercial SW release is rolled out.

### RAP SW update

In the trial release the SW for the RAP is always included in the VNF SW. Manual SW update with the RAP UI is required only for the RAP commissioning, while after the RAP is connected to a VNF, the VNF takes care the RAP is always running a correct SW version.

In addition to the RAP SW files deployed within the VNF SW, the user is also able to change the RAP SW version to be used with the VNF, and install new RAP SW files to the VNF with the CloudBTS WebUI.

### BTS profiles

In the trial release all supported BTS profiles are included in the CloudBTS VNF SW. As in bare-metal SBTS and in future commercial CloudBTS releases, in trial no separate BTS profile handling in the SW management is required.

Even if there is no management for the BTS profiles, there will still be several profiles included in the SW from which the user can select a correct one with a SCF parameter. E.g. for easier demonstrating of the scaling, a specific BTS profile can be selected to artificially limit the maximum number of cell allocations in one BB VM. This artificial limitation is available in trial only, and must be removed from the commercial release.

# Front-haul transport

# Radio network management functionality

In CloudBTS16 trial release the support of radio network management with NetAct is limited to CM, PM and FM. These fragments of the NE3S and the related NetAct adaptations are required in the PT2 sub-release only.

The rest of the NE3S fragments supported for bare-metal SBTS deployments will be introduced in the first commercial release as separate features.

The NE3S agent of the CloudBTS is implemented in the VNF part of the CloudBTS. The agent is part of the SiteOAM SW of the SBTS, and is deployed within a centralized VM of the CloudBTS, which does not scale. This means there will be only one NE3S agent, i.e. one NE3S connection for each CloudBTS in all CloudBTS deployments.

The V-SBTS instances in NetAct are not used for the CloudBTS configuration pre-validation, but instead the validation functionality of the SiteOAM in the CloudBTS can be used once the VNF is instantiated. The configuration validation is available in the CloudBTS VNF even if no RAPs are connected to it yet.

# Terms and Abbreviations

BTS:	A generic term. When a specific aspect of a BTS is referred, a more illustrative term (like SBTS, WBTS, bare metal BTS, etc) is used. Term BTS may refer to one RAT or to a more widely defined entity, like SBTS containing services for several RATs.

CAM:	Cloud Application Manager. CAM manages the VNFs in the data centres for the cloud. CAM provides user interface for VNF deployment and scaling operations.

Cloud BTS:	A cloud BTS consists of the virtual and physical parts of the BTS (SBTS VNF + Radio Access Point). In network topology an SBTS object represents the managed network element of the cloud BTS.

Cloud Server:	Also called as ‘Compute Node’ in some references. A cloud server is the physical HW used for the computing and storage services for the VNFs of the NEs. The servers are installed in server racks in the cloud data center. One cloud server may provide service for one or more VNF, or one VNF may occupy the capacity of several cloud servers.

Deployment:	In deployment the user adds a new VNF instance to the data center of the cloud. In RNW operability the concept of deployment can be extended to include the creation and integration of the virtual NE to the radio network topology. In deployment the related VMs with the mandatory basic NE functions are created, while additional VMs can later on be scaled out for completing the changing capacity or functionality requirements of the NE.

Edge Cloud:	Edge cloud is a cloud running in decentralized data center deployment. For BTSs the data center needs to be geographically close to the Radio Access Points (RAP), which leads to decentralized deployments of the data centres used for BTS VNF services.

Front-haul:	The southbound transport network between the VNF and its underlying radio network. In case of BTS the transport network between the cloud and the RAPs.

Guest OS:	The Guest OS is the operating system of the VNFC running on the VM provided for the VNFC service. The Guest OS belongs to the RCP.

Hypervisor:	SW layer providing the host services in a cloud for the guest VMs as virtual operating platforms.

Image server:	The image server or repository belongs to the VIM and provides the storage for the VM images needed to instantiate the VNFs in the data center.

LNBTS, LNCEL:	Radio objects in LTE system representing BTS and cell functions.

Mid-haul:	The communication path for the internal VNF communication between the cloud servers in the data center.

NCIR:	Nokia Cloud Infrastructure for Radio.

NASDA:	NetAct System Data Access (NASDA) is an entity in NetAct for storing the information of connected network elements. The data in NASDA includes the object type and identity of the NE, the NE version for adaptation, and the FQDN for resolving the IP address of the M-plane agent of the NE. NetAct applications are using NASDA to initiate communication with the NE in question.

Network Element:	Network Element (NE) refers here to a managed entity seen in the network topology in NetAct. A Network Element provides the same external management interface than a bare metal version of the NE. The external modelling of an NE does not change when it is virtualized.

NF:	Network Function. The NF represents the bare-metal deployment of the NE. VNF is a virtualized NF.

OAM Agent:	The OAM Agent is the O&M SW of the BTS RAP providing the operability services and communication towards the SBTS VNF.

Radio Access Point:	A Radio Access Point (RAP) is the physical radio device of the BTS. The RAP is a generic term including also the FZM connected to a FZ Controller, but in this document it refers to the physical part of a Cloud BTS. The RAP consists of the physical HW required for the radio including related antenna devices of the cells. One RAP represents the physical resources for one or more radio cells. There can be one or more RAPs connected to one SBTS VNF. The RAP can be based either on the FZM or FSM(x) HW. The term ‘point’ does not refer to single communication point, while an FSM(x) based RAP may have several communication points both in UE and SBTS VNF direction.

Radio Network Element:	Radio Network Element refers here to a logical network entity representing service of one radio technology. A Radio Network Element may be equal to the a Network Element, as in case of RNC, or one Network Element may include several Radio Network Elements – e.g. an SBTS may include one or more WBTS and/or eNB functions, which share the same M-plane agent of the SBTS .

As well as for the object for the NE, also the managed objects representing the Radio Network Elements are included in the RNW information model of the corresponding NE in NetAct

RCP:	Radio Cloud Platform. RCP is a common functionality provided for all Nokia radio network elements. As an exception, the BSC development has decided not to use RCP.

SBTS VNF:	A SBTS VNF is the virtual network function part of the Cloud BTS. The SBTS VNF is one VNF composed of VMs for the needed virtual components of the cloud BTS.

Scale IN:	Cloud resources for an existing radio network element are decreased by removing VNFC instances (VMs) from the VNF.

Scale OUT:	Cloud resources for an existing radio network element are increased by adding new VNFC instances (VMs) to the VNF.

Scale UP/DOWN:	In this scaling scenario the number of cores reserved for a VM is increased or decreased. This scaling scenario is not analysed yet (not in BTS trial)

Un-deployment:	In un-deployment the user initiates the removal of the VNF instance and related VMs from the data center. In RNW operability the concept of un-deployment can be extended to cover also the removal of the virtual NE from the RNW topology.

VIM:	Virtual Infrastructure Manager. With the help of the hypervisor the VIM executes the VNF operations from CAM and provides measurements and alarms from the infrastructure of the data center. In telco cloud for radio the VIM is provided by the NCIR product – Nokia Cloud Infrastructure for Radio.

Virtual NE:	The virtual NE here refers to the VNF of any network element in the Telco cloud. In case of SBTS the virtual NE term includes also the connected RAPs.

VM:	Virtual Machine. VM is the virtual processing and storage for the VNFC. A VM represents the virtualization container of the VNFC, meaning one VNFC runs on one VM.

VNF:	Virtual Network Function. In Radio Cloud the VNF represents one or more virtual network elements with one common M-plane termination point. A VNF consists of one or more VNF components (VNFC) with their virtual machines (VM). One VNF provides all the same NE services than the bare-metal version of the NE. In case of BTS, also the related WBTS or LNBTS functions are considered as network elements as they have dedicated external interfaces themselves, although they share the M-plane interface of the hosting SBTS.

VNFC:	Virtual Network Function Component. A virtual NE or VNF consists of one or more VNFC providing a specific portion of the NE service. One of the VNFCs of an NE runs the main O&M functionality of the VNF and provides a single RNW operability point for the virtual NE.

# References

| Identifier | Information |
| --- | --- |
| [SBTSArch] | https://sharenet-ims.inside.nokiasiemensnetworks.com/livelink/livelink/overview/D510220499 SBTS16 SW Architecture Specification |
| [NE3SSpecs] | https://sharenet-ims.inside.nokiasiemensnetworks.com/livelink/livelink?func=ll&objId=432131150&objAction=browse Folder containing NE3S material |
| [CloudBTSArch] | https://sharenet-ims.int.net.nokia.com/Overview/D530782303 Cloud BTS Architecture document |
| [TelcoCloud] | https://confluence.int.net.nokia.com/display/telcocloud/Telco+Cloud+Radio+Operability Telco Cloud Radio Operability confluence pages |
| [CAM] | https://confluence.int.net.nokia.com/display/CAM/Home CAM OpenStack confluence pages |
| [NCIR] | https://confluence.int.net.nokia.com/display/RCH/Architecture+and+feature+docs Radio Cloud Host / NCIR confluence pages |